פילטר בילטרלי משקלל פיקסלים סמוכים לפי שני גורמים:

- מרחק מרחבי: כמה הפיקסל רחוק מהפיקסל המרכזי.

- הבדל בעוצמת הבהירות: כמה ערך הפיקסל שונה מהפיקסל המרכזי.

שלבי מימושמימוש פונקציית Gaussian

כתוב את הנוסחה:

np.exp(-distance**2 / (2 * sigma**2))

לולאת עיבוד מרכזית

- עבור על כל פיקסל בתמונה.

- בחר חלון סביבו בגודל
window_size
 .

- לכל שכן:

  - חשב מרחק מרחבי (dx, dy).

  - חשב הבדל עוצמה (|I_center - I_neighbor| ).

  - חשב משקל מרחבי ומשקל עוצמה.

  - כפל ביניהם לקבלת המשקל הכולל.

  - בצע ממוצע משוקלל לפי משקלים אלו.

נירמול

חלק את הסכום המשוקלל בסכום המשקלים.המרה חזרה ל־uint8

לאחר סיום, חתוך לערכים בין 0–255. בדיקה

#!/usr/bin/env python3

# -*- coding: utf-8 -*-

"""

Bilateral Filter Implementation - Classroom Exercise

Image Processing Course - Spatial Operations

Basic edge-preserving smoothing filter

"""

import numpy as np

import time

from typing import Tuple

# Protection against using forbidden functions

FORBIDDEN_FUNCTIONS = ['bilateralFilter', 'edgePreservingFilter', 'jointBilateralFilter']

def check_forbidden_imports():

"""Check that no forbidden OpenCV functions are being used"""

import inspect

frame = inspect.currentframe()

if frame and frame.f_globals:

for forbidden in FORBIDDEN_FUNCTIONS:

if forbidden in str(frame.f_globals):

raise ValueError(f"Forbidden function {forbidden} detected! Use not allowed.")

def gaussian_weight(distance: float, sigma: float) -> float:

"""

Calculate Gaussian weight for given distance

Parameters:

-----------

distance : float

Distance value (spatial or intensity)

sigma : float

Standard deviation

Returns:

--------

float

Gaussian weight

"""

# TODO: Implement Gaussian weight calculation

# Hint: G(x) = exp(-x^2 / (2 * sigma^2))

pass

def bilateral_filter_basic(image: np.ndarray,

window_size: int = 5,

sigma_spatial: float = 3.0,

sigma_range: float = 50.0) -> np.ndarray:

"""

Basic implementation of bilateral filter

The bilateral filter is an edge-preserving smoothing filter that

combines spatial and intensity-based weighting.

Parameters:

-----------

image : np.ndarray

Input grayscale image (0-255)

window_size : int

Filter window size (must be odd, e.g., 3, 5, 7)

sigma_spatial : float

Spatial standard deviation (controls spatial extent)

sigma_range : float

Range/intensity standard deviation (controls edge preservation)

Returns:

--------

np.ndarray

Filtered image with same size and type as input

Raises:

-------

AssertionError

If parameters are invalid

"""

check_forbidden_imports()

# Input validation

assert window_size % 2 == 1, "Window size must be odd"

assert window_size >= 3, "Minimum window size is 3"

assert sigma_spatial > 0, "Spatial sigma must be positive"

assert sigma_range > 0, "Range sigma must be positive"

assert len(image.shape) == 2, "Input must be grayscale image"

# Convert to float64 to prevent overflow

img = image.astype(np.float64)

output = np.zeros_like(img)

# Image dimensions

height, width = img.shape

half_window = window_size // 2

# TODO: Implement bilateral filter

# Steps:

# 1. For each pixel (i,j) in the image:

#    a. Define the neighborhood window

#    b. For each neighbor pixel (k,l) in the window:

#       - Calculate spatial distance

#       - Calculate intensity difference

#       - Compute spatial weight using Gaussian

#       - Compute range weight using Gaussian

#       - Total weight = spatial_weight * range_weight

#    c. Normalize weights and compute filtered value

for i in range(height):

for j in range(width):

# TODO: Complete implementation

# Initialize weighted sum and weight sum

weighted_sum = 0.0

weight_sum = 0.0

# Get center pixel value

center_value = img[i, j]

# Process neighborhood

# TODO: Add your code here

# Store filtered value

output[i, j] = weighted_sum / weight_sum if weight_sum > 0 else center_value

return np.clip(output, 0, 255).astype(np.uint8)

def load_image(filename: str) -> np.ndarray:

"""

Load image from file (numpy array or simple formats)

Parameters:

-----------

filename : str

Path to image file

Returns:

--------

np.ndarray

Loaded image

"""

if filename.endswith('.npy'):

return np.load(filename)

else:

# For other formats, try to load as numpy array

try:

return np.loadtxt(filename, dtype=np.uint8)

except:

# Create a dummy image if loading fails

print(f"Warning: Could not load {filename}, using dummy image")

return np.random.randint(0, 256, (100, 100), dtype=np.uint8)

def save_image(image: np.ndarray, filename: str):

"""

Save image to file

Parameters:

-----------

image : np.ndarray

Image to save

filename : str

Output filename

"""

if filename.endswith('.npy'):

np.save(filename, image)

else:

np.savetxt(filename, image, fmt='%d')

def test_basic_implementation():

"""Test function for basic implementation"""

print("Testing basic bilateral filter implementation...")

# Create test image with noise

test_img = np.ones((100, 100), dtype=np.uint8) * 128

test_img[40:60, 40:60] = 200  # Add bright square

noise = np.random.normal(0, 20, test_img.shape)

noisy_img = np.clip(test_img + noise, 0, 255).astype(np.uint8)

# Apply filter

start_time = time.time()

filtered = bilateral_filter_basic(noisy_img)

elapsed = time.time() - start_time

print(f"Filtering completed in {elapsed:.3f} seconds")

print(f"Input range: [{noisy_img.min()}, {noisy_img.max()}]")

print(f"Output range: [{filtered.min()}, {filtered.max()}]")

# Basic validation

assert filtered.shape == noisy_img.shape, "Output shape mismatch"

assert filtered.dtype == np.uint8, "Output type should be uint8"

print("Basic tests passed!")

if __name__ == "__main__":

test_basic_implementation()